ESTIMATION FOR CONSTRUCTION OF BUILDING

NAME OF THE WORK: CONSTRUCTION OF SCHOOL BUILDING	LOCATION : BADVEL, KADAPA DT. ANDHRAPRADESH, INDIA

| ORGANIZATION: MOTHER TERESA MAHILA MANDALI |  |  | ADDRESS: 3/24 SURENDRA NAGAR BADVEL KADAPA DIST. A.P. INDIA 516227 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOUNDER & PRESIDENT: JINKA SANTHAMMA |  |  |  |  |  |  |  |  |  |  |
| S.NO. | PARTICULARS |  | NO.S | MEASUREMENT IN METERS |  |  | AREA m3 | RATE PER | AMOUNT IN |  |
|  |  |  |  | HEIGHT | LENGTH | WIDTH |  | m3 | RS. |  |
| 1 | Earth work excavation and depasting soil in initial lead and lift in b.c.Soils, red earth and ordinary gravel etc. |  |  |  |  |  |  |  |  |  |
|  | complete |  |  |  |  |  |  |  |  |  |
|  | columns |  | 1 x 22 | 1.50 | 1.50 | 1.80 | 89.10 |  |  |  |
|  | long walls |  | 1 x 2 | 16.50 | 0.60 | 0.60 | 11.88 | 80.00 | 8748.80 |  |
|  | cross walls |  | 1 x 2 | 11.65 | 0.60 | 0.60 | 8.38 |  |  |  |
|  |  |  |  |  |  |  | 103.36 |  |  |  |
| 2 | Sand filling and water ramming including the cost of conveyance of all materials and laborcharges in |  |  |  |  |  |  |  |  |  |
|  | foundation and basement in stanford specifications |  |  |  |  |  |  |  |  |  |
| a. | Foundation |  |  |  |  |  |  |  |  |  |
|  | columns |  | 22.00 | 1.50 | 1.50 | 0.30 | 14.85 |  |  |  |
|  | long walls |  | 1 x 2 | 16.50 | 0.60 | 0.15 | 2.95 |  | 33940.80 |  |
|  | cross walls |  | 1 x 2 | 11.65 | 0.60 | 0.15 | 2.09 | 160.00 |  |  |
| b. | Basement |  | 1 x 1 | 16.50 | 11.65 | 1.00 | 192.22 |  |  |  |
|  |  |  |  |  |  |  | 212.13 |  |  |  |
| 3 | P.C.C. in (1:4:8) to use 40 mm metal including the cost off conveyance of all materials and labor charges |  |  |  |  |  |  |  |  |  |
|  | etc. |  |  |  |  |  |  |  |  |  |
| a. | Foundation |  |  |  |  |  |  |  |  |  |
|  | columns |  | 1 x 22 | 1.50 | 1.50 | 0.30 | 14.85 |  |  |  |
|  | long walls |  | 1 x 2 | 1.50 | 1.50 | 0.15 | 2.97 | 1950.00 | 95043.00 |  |
|  | cross walls |  | 1 x 2 | 1.50 | 1.50 | 0.15 | 2.09 |  |  |  |
| b. | Basement |  | 1 x 1 | 16.50 | 11.65 | 0.15 | 28.83 |  |  |  |
| 4 | R.R. in c.m.(1:6) including the cost of conveyance of all materials and laborcharges |  |  |  |  |  |  |  |  |  |
|  | long walls |  | 1 x 2 | 14.75 | 0.60 | 1.60 | 28.32 | 1800.00 | 87264.00 |  |
|  | cross walls |  | 1 x 2 | 10.50 | 0.60 | 1.60 | 20.16 |  |  |  |
| 5 | R.C.C. (1:2:4) to use 40 mm metal including the cost and conveyance of all materials and labor charges etc |  |  |  |  |  |  |  |  |  |
| a. | Footings |  | 1 x 22 | 1.50 | 1.50 | 0.46 | 22.77 |  |  |  |
| b. | Columns |  | 1 x 22 | 0.35 | 0.23 | 8.00 | 14.16 | 3500.00 | 129255.00 |  |
| c | Plinth beam |  |  |  |  |  |  |  |  |  |
|  | Long beams |  | 1 x 2 | 16.50 | 0.23 | 0.35 | 2.65 |  |  |  |
|  | Cross Beams |  | 1 x 4 | 11.65 | 0.23 | 0.35 | 3.75 |  |  |  |
|  | Another beams |  | 1 x 4 | 5.30 | 0.23 | 0.35 | 1.70 |  |  |  |
|  | Another beams |  | 1 x 2 | 3.46 | 0.23 | 0.35 | 0.55 |  |  |  |
|  |  |  |  |  |  |  | 8.65 | 3800.00 | 32870.00 |  |

| d | Roof Beams |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Long beams | 1 x 2 | 16.50 | 0.23 | 0.35 | 2.65 |  |  |
|  | Cross beams | 1 x 4 | 11.65 | 0.23 | 0.35 | 3.75 |  |  |
|  | Another beams | 1 x 4 | 5.30 | 0.23 | 0.35 | 1.70 |  |  |
|  | Another Beams | 1 x 2 | 3.46 | 0.23 | 0.35 | 0.55 |  |  |
|  |  |  |  |  |  | 8.65 | 4000.00 | 34600.00 |
| e | Roof Slab | 1 x 1 | 16.50 | 11.65 | 0.10 | 19.22 | 4000.00 | 76880.00 |
| f | Stair case | 1 x 1 | 4.57 | 1.80 | 0.15 | 1.23 | 4000.00 | 4935.00 |
| 6 | SSF Steel grills including the cost and conveyance of all materials and labour charges |  |  |  |  |  |  |  |
| a | Columns |  | 14.16 x 130 Kgs. |  |  | 1840.00 |  |  |
| b | Footings |  | 22.77 x 100 kgs |  |  | 2277.00 |  |  |
| c | Plinth beam |  | 8.65 x 120 kgs |  |  | 1038.00 |  |  |
| d | Roof Beams |  | 8.65 x 120 kgs |  |  | 1038.00 | 42.00 | 338898.00 |
| e | Roof slab |  | 19.22 X 90 Kgs. |  |  | 1729.00 |  |  |
| f | Stair case |  | 1.23 x 120 Kgs. |  |  | 147.00 |  |  |
|  |  |  |  |  |  | 8069.00 |  |  |
| 7 | Brick Masonary C.M. (1:6) including the cost of transportation & labor charges |  |  |  |  |  |  |  |
|  | Long walls | 1 x 2 | 16.50 | 0.23 | 2.75 | 2.75 |  |  |
|  | Cross walls | 1 x 4 | 11.65 | 0.23 | 2.75 | 2.75 |  |  |
|  | Another walls | 1 x 6 | 4.67 | 0.23 | 2.75 | 17.72 |  |  |
|  | Another walls | 1 x 2 | 3.00 | 0.23 | 2.75 | 3.79 |  |  |
|  | Deductions |  |  |  |  | 71.85 | 1900.00 | 118693.00 |
|  | D.Doors | 1 x 8 | 1.00 | 0.23 | 2.10 | 3.86 |  |  |
|  | W Windows | 1 x 20 | 1.00 | 0.23 | 1.20 | 5.52 |  |  |
|  |  |  |  |  |  | 9.38 |  |  |
|  | Net Quantity |  | 71.85 - | 9.38 |  | 62.47 |  |  |
| 8 | Supply and fixing of doors and windows including transportation & labor charges |  |  |  |  |  |  |  |
|  | Deductions |  |  |  |  |  |  |  |
|  | D Doors | 1 x 8 | 1.00 |  | 2.10 | 16.80 |  |  |
|  | W Windows | 1 x 20 | 1.00 |  | 1.20 | 24.00 | 3200.00 | 130560.00 |
|  |  |  |  |  |  | 40.80 |  |  |
| 9 | Plastering with C.M.(1:4) 20mm thick including labor and transportation charges |  |  |  |  |  |  |  |
|  | long walls | 1 x 4 | 16.50 |  | 3.10 | 204.60 |  |  |
|  | cross walls | 1 x 8 | 11.65 |  | 3.10 | 288.92 |  |  |
|  | Another walls 1 | 1 x 12 | 4.67 |  | 3.10 | 173.72 | 120.00 | 84532.80 |
|  | Another walls 2 | 1 x 4 | 3.00 |  | 3.10 | 37.20 |  |  |
|  |  |  |  |  |  | 704.44 |  |  |

| 10 | Flooring with vertificient tiles in required sizes 10 cms thick 40 mm H.B.G. metal including transportation & |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | labor charges |  |  |  |  |  |  |  |  |
|  | Class Rooms | 1 x 6 | 3.00 x 4.67 |  |  | 84.06 |  |  |  |
|  | Office Room | 1 x 1 | 3.00 x 3.00 |  |  | 9.00 |  | 800.00 | 81648.00 |
|  | Staff Room | 1 x1 | 3.00 x 3.00 |  |  | 9.00 |  |  |  |
|  |  |  |  |  |  | 102.06 |  |  |  |
| 11 | Colour wash with two coats including the labor & transportation costs |  |  |  |  | 19.22 |  |  |  |
|  |  |  |  |  |  |  |  | 70.00 | 50656.20 |
|  |  |  |  |  |  | 704.44 |  |  |  |
|  |  |  |  |  |  | 723.66 |  |  |  |
|  |  |  |  |  |  | Sub Total |  |  | 1308522.00 |
| 12 | Provision for Electrical Charges@ 7.5 % |  |  |  |  |  |  |  | 98139.00 |
|  |  |  |  |  | TOTALGROUND |  | FLOOR |  | 1406661.00 |

|  | FIRST FLOOR |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | R.C.C. ( 1:2:4) to use 20 mm metal including labor & transportation costs |  |  |  |  |  |  |  |
| a | Columns | 1 x 22 | 0.35 | 0.23 | 2.75 | 14.16 | 4500.00 | 63720.00 |
| b | Roof beams |  |  |  |  |  |  |  |
|  | Long Beams | 1 x 2 | 16.50 | 0.23 | 0.35 | 2.65 |  |  |
|  | Cross beams | 1 x 4 | 11.85 | 0.23 | 0.35 | 3.75 |  |  |
|  | Another Beams | 1 x 4 | 5.30 | 0.23 | 0.35 | 1.70 | 4500.00 | 38954.00 |
|  | Another Beams | 1 x 2 | 3.46 | 0.23 | 0.35 | 0.55 |  |  |
|  |  |  |  |  |  | 8.65 |  |  |
| c | Roof Slab | 1 x 1 | 16.50 | 11.65 | 0.10 | 19.22 | 4500.00 | 86490.00 |
| 2 | SSF Steel grills including labor & transportation costs |  |  |  |  |  |  |  |
| a | Columns |  |  | 14.16 x 160 kgs |  |  | 42 per Kg |  |
| b | Roof Beams |  |  | 8.65 x 120 Kgs |  |  | 1038 per Kg | 193494.00 |
| c | Roof Slab |  |  | 19.22 x 90 Kgs |  |  | 1729 per Kg |  |
| 3 | Brick masnary C.M. (1:6) including labor & transportation costs |  |  |  |  |  |  |  |
|  | Long Beams | 1 x 2 | 16.50 | 0.23 | 2.75 | 20.82 |  |  |
|  | Cross walls | 1 x 4 | 11.65 | 0.23 | 2.75 | 39.47 |  |  |
|  | Another walls | 1 x 6 | 4.67 | 0.23 | 2.75 | 17.72 |  |  |
|  |  | 1 x 4 | 3.00 | 0.23 | 2.75 | 3.79 |  |  |
|  |  |  |  |  |  | 71.85 | 2100 | 131187.00 |
|  | Deductions |  |  |  |  |  |  |  |
|  | D Doors | 1 x 8 | 1.00 | 0.23 | 2.10 | 3.86 |  |  |
|  | Windows | 1 x 20 | 1.00 | 0.23 | 1.20 | 5.52 |  |  |
|  |  |  |  |  |  | 9.32 |  |  |
| 4 | Supply and fixing of doors and windows including transportation & labor charges |  |  |  |  |  |  |  |
|  | Deductions |  |  |  |  |  |  |  |
|  | D Doors | 1 x 8 | 1.00 |  | 2.10 | 16.80 |  |  |
|  | Windows | 1 x 20 | 1.00 |  | 1.20 | 24.00 | 3200.00 | 130560.00 |
|  |  |  |  |  |  | 40.80 |  |  |
| 5 | Plastering with C.M.(1:4) 20mm thick including labor and transportation charges |  |  |  |  |  |  |  |
|  | long walls | 1 x 4 | 16.50 |  | 3.10 | 204.60 |  |  |
|  | cross walls | 1 x 8 | 11.65 |  | 3.10 | 288.92 |  |  |
|  | Another walls 1 | 1 x 12 | 4.67 |  | 3.10 | 173.72 | 130.00 | 91577.00 |
|  | Another walls 2 | 1 x 4 | 3.00 |  | 3.10 | 37.20 |  |  |
|  |  |  |  |  |  | 704.44 |  |  |
|  | Flooring with vertificient tiles in required sizes 10 |  |  |  |  |  |  |  |
| 6 | cms thick 40 mm H.B.G. metal including | 1 x 1 | 21.00 |  |  | 102.75 | 850.00 | 87337.00 |
|  | transportation & labor charges |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Sub Total |  | 823319.00 |
| 7 | Electrification charges @ 7.5% |  |  |  |  |  |  | 61748.00 |
|  |  |  | T O T A L FIRST FLOOR |  |  |  |  | 885067.00 |

|  | SECOND FLOOR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | R.C.C. (1:2:4) to use 20 mm metal including labor |  |  |  |  |  |  |  |  |  |
|  | & transportation costs |  |  |  |  |  |  |  |  |  |
| a | Columns | 1 x 8 | 2.75 |  |  | 0.23 | 0.35 | 1.77 |  |  |
| b | Long Beam | 1 x 2 | 7.85 |  |  | 0.23 | 0.35 | 1.25 |  |  |
| c | Cross Beam | 1 x4 | 4.67 |  |  | 0.23 | 0.35 | 1.25 | 4800.00 | 39180.00 |
| d | Roof Slab | 1 x 1 | 7.80 |  |  | 4.67 | 0.10 | 3.60 |  |  |
|  |  |  |  |  |  |  |  | 8.16 |  |  |
| 2 | SSF Steel grills including labor & transportation costs |  |  |  |  |  |  |  |  |  |
|  | Columns |  | 1.77 x 130 kgs |  |  |  |  | 230 kgs |  |  |
|  | Roof Beams |  | 2.75 x 120 kgs |  |  |  |  | 330 kgs | 42.00 | 37279.00 |
|  | Roof Slab |  | 3.64 x 90 kgs |  |  |  |  | 887 kgs |  |  |
| 3 | Brick masnary C.M. (1:6) including labor & transportation costs |  |  |  |  |  |  |  |  |  |
|  | Long walls | 1 x 2 | 7.80 |  |  | 0.23 | 2.75 | 9.87 |  |  |
|  | Cross Walls | 1 x 4 | 4.67 |  |  | 0.23 | 2.75 | 11.82 |  |  |
|  |  |  |  |  |  |  |  | 21.67 |  |  |
|  | Deductions |  |  |  |  |  |  |  | 2300.00 | 45747.00 |
|  | D Doors | 1 x 2 | 1.00 |  |  | 0.23 | 2.10 | 0.96 |  |  |
|  | W Windows | 1 x 3 | 1.00 |  |  | 0.23 | 1.20 | 0.82 |  |  |
|  |  |  |  |  |  |  |  | 1.78 |  |  |
|  | Net Quantity |  | 21.67 - 1.78 = 19.89 |  |  |  |  |  |  |  |
| 4 | Supplying & Fixing of Doors Windows including labor & transportation costs |  |  |  |  |  |  | 7.80 | 3200.00 | 24960.00 |
| 5 | Plastering with C.M. (1:4) 20 mm thick including labor & transportation costs |  |  |  |  |  |  |  |  |  |
|  | Long walls | 1 x 4 | 7.80 |  |  |  | 3.10 | 96.72 |  |  |
|  | Cross Walls | 1 x 8 | 4.67 |  |  |  | 3.10 | 115.81 | 150.00 | 31879.00 |
|  |  |  |  |  |  |  |  | 212.53 |  |  |
| 6 | Flooring with Rajasthan Marbles including labor & | 1 x 2 | 4.67 |  |  | 3.00 |  | 28.02 | 1000.00 | 28000.00 |
|  | transportation costs |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Sub | Total | 207045.00 |
| 7 | Electrification Charges @7.5 % |  |  |  |  |  |  |  |  | 20000.00 |
| 8 | Provision for Sanitary and water supply |  |  |  |  |  |  |  |  | 120000.00 |
|  | arrangement |  |  |  |  |  |  |  |  |  |
| 9 | Provision for staircase with grills |  |  |  |  |  |  |  |  | 60000.00 |
| 10 | Provison for Balcony Grills |  |  |  |  |  |  |  |  | 100000.00 |
| 11 | Provision for Septic Tank |  |  |  |  |  |  |  |  | 20000.00 |
|  |  |  |  |  |  | TOTAL | second floor |  |  | 527045.00 |

ABSTRACT

|  | in Rs. | in $ |
| --- | --- | --- |
| GROUND FLOOR | 1406661.00 | 29928.96 |
| FIRST FLOOR | 885067.00 | 18831.21 |
| SECOND FLOOR | 527045.00 | 11213.72 |
|  | 2818773.00 | 59973.89 |